dvItemNumberMasked	dvSubjectWithSoftReturns (dvFileNumber)

dvDivisionName

dvMasterProgramItem1

## Executive Summary

The application seeks approval for the construction of buildings and works comprising the development of the land for seven buildings (three 5 storey buildings and four 6 storey buildings over 2 basement levels), containing 603 dwellings, a 24 hour supermarket, 17 shops, 3 food and drink premises and 5 offices, road works, use of the land for dwellings and a reduction of the car parking requirements.

The proposed development and use is generally in accordance with the East Brunswick Village (EBV) Development Plan endorsed by Council Officers under delegation on 4 October 2012. The endorsed EBV Development Plan details building envelopes for the site to a maximum of 6 storeys, and requirements to provide public realm works, a new street with signalised intersection at Nicholson Street and developer contributions. These have been met or agreed to under the permit application.

Under Clause 43.04-2 of the Moreland Planning Scheme, the application is exempt from public notice as it is generally in accordance with the endorsed EBV Development Plan. No objections have been received.

The key planning issue under this application relates to the internal amenity in apartments proposed to be located directly adjacent to existing heavy industry. The recommendation includes staging the development, to preclude endorsement of plans and development of apartments in locations where existing industrial impacts cannot be acceptably mitigated through design. Endorsement of the affected apartments could occur at a later date when the heavy industry vacates or impacts can otherwise be demonstrated to be adequately addressed through mitigation.

The report details the assessment of the application against the policies and provisions of the Moreland Planning Scheme.

The Group Manager City Development is recommending the issue of a Planning Permit for the proposal subject to conditions.

Do not delete this line

## Recommendation - The Urban Planning Committee resolve: - That a Planning Permit No. MPS/2013/979 be issued for the staged construction of buildings and works comprising the development of the land for seven buildings (three 5 storey buildings and four 6 storey buildings over 2 basement levels), containing multiple dwellings, a supermarket, shops, food and drink premises and offices, use of the land for dwellings, creation of access to a Road Zone, Category 1, and a reduction of the car parking requirements at 127-137, 139 & part 149 Nicholson Street, Brunswick East, subject to the following conditions: - Staging - Prior to the endorsement of plans for Stage 1, staging plans to the satisfaction of the Responsible Authority must be submitted to and approved by the Responsible Authority. When approved, the staging plans will be endorsed and will then form part of the permit. The plans must be drawn to scale with dimensions and three copies must be provided. The plans must be generally in accordance with the staging shown on plans prepared by JAM Architects Pty Ltd, Sheet Nos. TP05 and TP12 to TP20 inclusive, Revision D, dated 26 November 2014, Council date-stamped 4 December 2014, but modified to show: - a) The extent of Stage 1 and 2 buildings and works clearly labelled ‘Stage 1’ and ‘Stage 2’ and hatched (or otherwise clearly differentiated). - b) Sheet Nos. TP05, TP13 and TP14 annotated with the words: ‘The John Street basement vehicle access points and the west basement are to be developed as part of Stage 1, but are not to be used prior to the commencement of the residential use in Stage 2.’ - c) Each plan annotated with the words: ‘This plan has been endorsed to detail staging only and does not constitute endorsement of design details, including but not limited to, floor layouts, setbacks, road works, landscaping and open space.’ - The staging as shown on the endorsed staging plans must not be altered without the written consent of the Responsible Authority. - Stage 1 - Amended Plans - Before the use or development of Stage 1 commences, amended plans to the satisfaction of the Responsible Authority must be submitted to and approved by the Responsible Authority. When approved, the plans will be endorsed and will then form part of the permit. The plans must be drawn to scale with dimensions and three copies must be provided. The plans must be generally in accordance with the plans prepared by JAM Architects Pty Ltd, Sheet Nos. TP02 to TP40 inclusive, TP62, TP65, TP68, TP72 and TP74, Revision D, dated 26 November 2014, Council date-stamped 4 December 2014, but modified to show: - A development summary confined to Stage 1 only. - Deletion of all buildings and works relevant to Stage 2 under this permit. - Fencing and planter box landscaping to the south, east and west perimeter of Lot 4 generally in accordance with the plan prepared by JAM Architects Pty Ltd, Sheet No. TP05, Revision D.1 dated 28 January 2015. - The interim and ultimate road reserve boundaries clearly delineated and annotated on basement, ground and all upper floor plans. The plans must include dimensioned building setbacks from the interim and ultimate road reserve boundaries at each level. - Deletion of annotations relevant to Rickard Street: ‘footpath to be adjusted to council detail’; and ‘mountable kerb to be installed to council detail’. - Windows in the north-facing bedroom elevations to Units 1.15, 2.15, 3.14 and 4.14 of Lot 1, or reconfiguration of these dwellings to include north-facing living areas. - Windows in the east-facing kitchen / living room elevations to Units 1.13, 2.13, 3.13 and 4.13 of Lot 3. - Windows in the south-facing kitchen / living room elevations to Units 1.14 and 2.14 of Lot 3. - A minimum 1.7 metre high wall above finished floor level at the southern end of the first floor common internal courtyard to Lot 2, extending to screen the southern balcony perimeters to Units 1.19 and 1.32. - A minimum 1.7 metre high wall or screening at the southern end of the bedroom balcony to Unit 1.31 of Lot 2. - A minimum 1.7 metre high wall or screening at the southern end of the living room balcony to Unit 2.32 of Lot 2. - Obscure glazing to a minimum height of 1.7 metres to the south-facing windows in the common circulation corridors at first and second floor levels to Lot 2 adjacent to Units 1.31 and 1.32, and Units 2.31 and 2.32. - Transparent or semi-transparent (visually permeable) finishes to the ground floor north side stairwells of Lot 2. - Operable / openable windows within common residential circulation corridors to Lots 1 and 2, or expert advice detailing why this cannot be practically achieved from a registered building surveyor or other suitably qualified person. - Effective shading to the exposed north and west facing habitable room windows, where not already shown, including design details at 1:50 or 1:100 which differentiate between effective shading required to provide thermal comfort to each façade / orientation. Lower level windows which will be shaded from buildings within or adjacent to the development are exempted from this requirement where it can be satisfactorily demonstrated. - Effective shading to the exposed east facing bedroom windows to Units 3.01, 3.03, 3.05, 4.01, 4.03 and 4.05 of Lot 1 and Units 3.14, 3.16, 3.19, 4.14, 4.16 and 4.19 of Lot 3, including design details at 1:50 or 1:100 to provide thermal comfort relevant to the eastern orientation. - Basement water tank capacities. - The retail verandahs setback not less than 750mm from the edge of the new ‘Main Street’ carriageway and the proposed kerbline relevant to the ultimate Nicholson Street road reserve. Verandahs are to be clear of any street trees. - An intersection threshold over the access road between Lots 3 and 4 generally in keeping with the threshold / treatment between Lots 4 and 6. - Any modifications to the plans arising from the Acoustic Report in accordance with Condition 8 of this permit. - Any modifications to the plans arising from the amended Sustainable Design in the Planning Process Plan in accordance with Condition 26 of this permit. - Any modifications to the plans arising from the Access Plan in accordance with Condition 29 of this permit. A minimum of 29 adaptable dwellings or ten percent of the total number of dwellings, (whichever is greater), must be shown on the floor plans (not to be provided as optional). - Any modifications to the plans arising from the amended Waste Management Plan in accordance with Condition 31 of this permit. - Any modifications to the plans arising from the amended Landscape Plan in accordance with Condition 34 of this permit. - A sample board of all proposed exterior decorations, materials, finishes and colours, including colour samples, and 3 copies of a schedule in a form that can be endorsed and filed. - Cessation of Commercial Laundry - Prior to the commencement of the development of Stage 1, excluding any works that necessarily form part of the Environmental Audit process, Planning Permit MPS/2005/65 issued on 19 October 2005, and amended on 25 January 2007, for the use of the land at 139-149 Nicholson Street, Brunswick East, as a service industry (commercial laundry), must be cancelled, and written confirmation of the cancellation provided to the satisfaction of the Responsible Authority. - Removal of Easement - Prior to the commencement of works of Stage 1, the carriageway easement E3 on Plan of Subdivision PS 537189G must be removed from the Certificate of Title Vol. 10921 Fol. 714. A title search showing the removal of the carriageway easement must be submitted to the Responsible Authority to demonstrate compliance with this condition. - Boundary Realignment / Consolidation of Titles - Unless with the prior written consent of the Responsible Authority, prior to the occupation of Stage 1, or within 12 months of the date of issue of this permit, whichever comes first, titles to the land must be consolidated, and/or boundaries realigned, and certified plans lodged with the Land Registry Office, at no cost to and to the satisfaction of the Responsible Authority, to eliminate any land-locked parcels. - Interim perimeter fencing and planting to Lot 4 - Prior to the occupation of any part of Stage 1 of the development, the perimeter fencing and planter box landscaping to Lot 4 must be constructed / installed to the satisfaction of the Responsible Authority. - Acoustic Attenuation - Prior to the endorsement of plans for Stage 1, an Acoustic Report must be prepared by a suitably qualified acoustic engineer or equivalent to the satisfaction of the Responsible Authority. The Report must prescribe specific attenuation measures / treatments, including design details where applicable, to ensure dwelling occupants of Lots 1, 2 and 3 are provided with an acceptable level of internal amenity in respect to external noise emanating from the Nicholson Street arterial road and the proposed supermarket. When submitted and approved to the satisfaction of the Responsible Authority, the Acoustic Report and associated notated plans will be endorsed to form part of this permit. - The recommendations of the report must be implemented to the satisfaction of the Responsible Authority prior to the occupation of Stage 1 of the development. No alterations to the Acoustic Report may occur without the written consent of the Responsible Authority. - Road Safety Audit – Internal Roads - Prior to the commencement of the detailed design stage for the internal roads, preliminary design plans for the internal roads must be subjected to an Independent Preliminary Design Stage Road Safety Audit by a suitably qualified independent auditor in accordance with Austroads ‘Road Safety Audit’ (2002). The findings of the audit must be resolved by the applicant to the satisfaction of the Responsible Authority. - Prior to the commencement of construction of the internal roads, the detailed design plans must be subjected to an Independent Detail Design Stage Road Safety Audit by a suitably qualified independent auditor in accordance with Austroads ‘Road Safety Audit’ (2002). The findings of the audit must be resolved by the applicant to the satisfaction of the Responsible Authority. - Prior to the commencement of the use of the internal roads, the internal roads must be subjected to an Independent Pre-Opening Stage Road Safety Audit by a suitably qualified independent auditor in accordance with Austroads ‘Road Safety Audit’ (2002). The findings of the audit must be resolved by the applicant prior to the occupation of any part of the development to the satisfaction of the Responsible Authority. - All Road Safety Audits must consider the operation of the shopping centre as a whole, including interactions with the surrounding street and footpath network. - Loading / Unloading - Prior to the commencement of the supermarket use, a Supermarket Loading and Unloading Management Plan must be prepared by a suitably qualified consultant to the satisfaction of the Responsible Authority. The Plan must include, but not be limited to, the following: - a) Delivery hours restricted to: - i. 6am to 11pm Monday to Saturday; and - ii. 8am to 11pm Sunday and public holidays. - b) Conditions of loading arrangements, including measures taken to prevent deliveries from being made outside of permitted hours. - c) Delivery vehicles restricted to vehicles having a length of no greater than 12.5 metres. - d) Measures to prevent conflict with pedestrians, cyclists and vehicles. - e) Turning circles at a scale of 1:200 for 12.5 metre long single unit trucks extending into Rickard Street, and associated extent of parking bans required in Rickard Street to facilitate truck access. When submitted and approved to the satisfaction of the Responsible Authority, the Supermarket Loading and Unloading Management Plan will be endorsed to form part of this permit. - The Supermarket Loading and Unloading Management Plan must be implemented and complied with at all times to the satisfaction of the Responsible Authority. No alterations to the Plan may occur without the written consent of the Responsible Authority. - All deliveries to Lots 1 or 2 via Rickard Street are restricted to vehicles having a length of no greater than 12.5 metres. - Stage 2 - Amended Plans - Before the use or development of Stage 2 commences, amended plans to the satisfaction of the Responsible Authority must be submitted to and approved by the Responsible Authority. When approved, the plans will be endorsed and will then form part of the permit. The plans must be drawn to scale with dimensions and three copies must be provided. The plans must be generally in accordance with the plans prepared by JAM Architects Pty Ltd, Sheet Nos. TP02 to TP24 inclusive, TP41 to TP61, TP63 to TP65, TP68, TP72, TP74 to TP75, Revision C, dated 22 May 2014, Council date-stamped 6 June 2014, but modified to show: - a) Any modifications to the plans arising from the Adverse Amenity Report in accordance with Condition 17 of this permit to satisfactorily address adverse amenity impacts of noise and vibration on the future residents of Lots 4, 6 and 7 from nearby Commercial, Industrial or Trade Premises without: - i. The need to construct a solid high screen wall (acoustic barrier) or otherwise modify the design in a way which would result in unreasonable impacts on internal or external amenity (e.g. in terms of ventilation, outlook and bulk). - ii. The development ceasing to be generally in accordance with the East Brunswick Village (EBV) Development Plan endorsed by Council on 4 October 2012, or as otherwise amended. - b) A development summary confined to Stage 2 only, and for Stage 1 and 2 combined. - c) All buildings and works relevant to Stage 2 under this permit differentiated from buildings and works relevant to Stage 1. - d) The west-facing balcony balustrades above ground floor level to Lots 5 and 7, excluding those facing the internal courtyard to Lot 7, finished in a solid non-transparent material, or a partially transparent material if utilising patterning / images or lasercut technology. - e) The south-facing windows to Units 1.24 and 1.25 of Lot 5 obscure glazed and having maximum awning window openings of 125mm, or otherwise designed to minimise overlooking of adjacent private open space and habitable room windows within 9 metres of 7 and 9 Gamble Street. - f) A window or windows in the west-facing bedroom 1 elevation to Unit 5.05 of Lot 4. - g) A window or windows in the east-facing kitchen / living room elevation to Unit 5.10 of Lot 4. - h) Windows in the north-facing bedroom elevations to Units 0.01, 1.01, 2.01 and 3.01 of Lot 5, or reconfiguration of these dwellings to include north-facing living areas. - i) A window or windows in the north-facing bedroom 1 elevation to Unit 4.01 of Lot 5, deletion of the west-facing bedroom 1 window to Unit 4.01 of Lot 5, or reconfiguration of this dwelling to include a north-facing living area. - j) Windows in the east-facing study elevations to Units 0.13 and 2.13 of Lot 5. - k) A window or windows in the south-facing study elevation to Unit 0.23 of Lot 5. - l) Windows in the south-facing bedroom 1 elevations to Units 2.23 and 3.23 of Lot 5. - m) Windows in the south-facing kitchen / living room elevations to Units 4.18 and 5.10 of Lot 5. - n) Windows in the south-facing bedroom 2 elevations to Units 4.19 and 5.11 of Lot 5. - o) A window or windows in the west-facing bedroom 1 elevation to Unit 5.01 of Lot 6. - p) A window or windows in the east-facing kitchen / living room elevation to Unit 5.03 of Lot 6. - q) A window or windows in the east-facing bedroom 1 elevation to Unit 0.08 of Lot 7. - r) A dimensioned 2 metre (minimum) ground and first floor south setback to Lot 5 from 9 Gamble Street. - s) Deletion of the northern-most planter box to the east of Lot 7 to accommodate Metropolitan Fire Brigade (MFB) fire truck emergency access requirements in accordance with the swept path analysis included in the Traffic and Transport Assessment prepared by Cardno Victoria Pty Ltd dated 27 March 2013. - t) Deletion of the bollards at the western end of ‘Main Street’ adjacent to John Street to facilitate emergency vehicle access. - u) Deletion of the crossover on John Street at the western end of ‘Main Street’. - v) The 24 hour public access plan including the 2 metre wide setback area directly north of Lot 7. - w) Operable / openable windows within common residential circulation corridors to Lots 4, 5, 6 and 7, or expert advice detailing why this cannot be practically achieved from a registered building surveyor or other suitably qualified person. - x) Effective shading to the exposed north and west facing habitable room windows, where not already shown, including design details at 1:50 or 1:100 which differentiate between effective shading required to provide thermal comfort to each façade / orientation. Lower level windows which will be shaded from buildings within or adjacent to the development are exempted from this requirement where it can be satisfactorily demonstrated. - y) The retail verandahs setback not less than 750mm from the edge of the new ‘Main Street’ carriageway. Verandahs are to be clear of any street trees. - z) Any modifications to the plans arising from the Public Works Plan in accordance with Condition 17 of this permit. - aa) Any modifications to the plans arising from the amended Sustainable Design in the Planning Process Plan in accordance with Condition 26 of this permit. - bb) Any modifications to the plans arising from the Access Plan in accordance with Condition 29 of this permit. A minimum of 31 adaptable dwellings or ten percent of the total number of dwellings, (whichever is greater), must be shown on the floor plans (not to be provided as optional). - cc) Any modifications to the plans arising from the amended Waste Management Plan in accordance with Condition 31 of this permit. - dd) Any modifications to the plans arising from the amended Landscape Plan in accordance with Condition 34 of this permit. - ee) A sample board of all proposed exterior decorations, materials, finishes and colours, including colour samples, and 3 copies of a schedule in a form that can be endorsed and filed. - Adverse Amenity Impacts - Prior to the endorsement of plans for Stage 2 of the development, an Adverse Amenity Report must be prepared by a suitably qualified consultant to the satisfaction of the Responsible Authority. The Report must address adverse amenity impacts (noise and vibration) and prescribe specific attenuation measures / treatments, including design details where applicable, to ensure dwelling occupants of Lots 4, 6 and 7 are provided with an acceptable level of amenity in respect to external noise and vibration. - The report must satisfy Council that the occupation of Lots 4, 6 and 7 for residential purposes would not result in a breach of SEPP N-1 by nearby Commercial, Industrial or Trade Premises. - Any attenuation measures / treatments to address this condition must not include the construction of a solid high screen wall (acoustic barrier) or otherwise require the design to be modified in a way which would result in unreasonable impacts on internal or external amenity (e.g. in terms of ventilation, outlook and bulk). When submitted and approved to the satisfaction of the Responsible Authority, the Adverse Amenity Report and associated notated plans will be endorsed to form part of this permit. - The recommendations of the report must be implemented to the satisfaction of the Responsible Authority prior to the occupation of Stage 2 of the development. No alterations to the Adverse Amenity Report may occur without the written consent of the Responsible Authority. Public Works Plan – John Street - Prior to the endorsement of plans for Stage 2 of the development, a Public Works Plan must be developed in collaboration with the Responsible Authority, submitted and approved to the satisfaction of the Responsible Authority, detailing streetscape improvement works to the John Street frontage. The Plan must include consideration of the planting of street trees, the replacement of the existing footpath, nature strip and redundant vehicle crossings, the replacement and relocation of any services and the installation of street furniture. When approved, the Public Works Plan will form part of the endorsed plans under this permit. The endorsed Plan must be implemented prior to the occupation of Stage 2 of the development unless otherwise agreed in writing by the Responsible Authority. All works associated with the endorsed Plan must be undertaken at no cost to Council. - John Street Road Upgrades - Prior to the issue of the first Statement of Compliance for any subdivision application within Stage 2 of this development, or within five (5) years of the date of issue of this permit, whichever comes first, a road hump is to be installed in John Street, and intersection thresholds installed at Glenlyon Road / John Street and Albert Street / John Street in accordance with the Development Plan approved by Council on 4 October 2012, or as otherwise amended. All works are to be undertaken to the satisfaction of the Responsible Authority, under the supervision of the Responsible Authority, and at no cost to Council. - Parking - Prior to the occupation of Stage 2 of the development, the residential car parking spaces provided on the land must be marked with the associated residential apartment number to facilitate management of the car park to the satisfaction of the Responsible Authority. Section 173 Agreement – Restrictions on Subdivision and Development and Use of Stage 2 - Within 12 months of the date of issue of this permit, an agreement under Section 173 of the Planning and Environment Act 1987 must be entered into between the owner/s of the land and the Responsible Authority in a form satisfactory to the Responsible Authority to provide for: - a) Unless otherwise agreed by the Responsible Authority in writing, no subdivision of the land, or any part of the land within the area identified as Stage 2 under the endorsed Staging Plan pursuant to Condition 1 of this permit, may occur until such time that plans have been endorsed for Stage 2 pursuant to Condition 15 of this permit. - b) Endorsement of plans including any residential component within Lots 4, 6 and 7 (as identified on the endorsed Staging Plan pursuant to Condition 1 of this permit) and construction of any residential component within Lots 4, 6 and 7 cannot occur unless the Responsible Authority is first satisfied in writing that: - i. Adverse amenity impacts of noise and vibration on the future residents of Lots 4, 6 and 7 from nearby Commercial, Industrial or Trade Premises have been resolved or satisfactorily addressed, without the need to construct a solid high screen wall (acoustic barrier) or otherwise modify the design in a way which would result in unreasonable impacts on internal or external amenity (e.g. in terms of ventilation, outlook and bulk); - ii. Any off-site mitigation undertaken ‘at the source’ to address adverse amenity impacts has been completed and evidence provided; and - iii. The occupation of Lots 4, 6 and 7 for residential purposes would not result in a breach of SEPP N-1 by nearby Commercial, Industrial or Trade Premises. - Prior to the commencement of the development of Stage 1 approved under this permit, application must be made to the Registrar of Titles to Register the Section 173 Agreement on the title to the land under Section 181 of the Planning and Environment Act 1987. - The owner / permit applicant of the land subject of this permit must pay all of Council’s legal costs and expenses of this agreement, including preparation, execution and registration on title. Section 173 Agreement - Community Infrastructure Contribution - Within 12 months of the date of issue of this permit, an agreement under Section 173 of the Planning and Environment Act 1987 must be entered into between the owner/s of the land and the Responsible Authority in a form satisfactory to the Responsible Authority to provide for: - a) Payment of a Community Infrastructure cash contribution of $205.64 (“The Contribution”) per dwelling in accordance with the Moreland Development Contributions Plan January 2015, (as may be amended by Council), prior to the issue of a Building Permit under the Building Act 1993 and the relevant Building Regulations for each Stage of the development, or at any other time set out in an agreement with the Moreland City Council as the Collecting Agency. - b) Adjustment of the cash contribution rate of $205.64 per dwelling (calculated as at 1 July 2012), shall occur on 1 July each year, generally in accordance with the Moreland Development Contributions Plan January 2015. If the Development Contributions Plan does not form part of the Moreland Planning Scheme at the date the Contribution is first payable by the Owner, the Contribution shall be adjusted to reflect CPI increases from 1 July 2012. - Prior to the development commencing, application must be made to the Registrar of Titles to Register the Section 173 Agreement on the title to the land under Section 181 of the Planning and Environment Act 1987. - The owner / permit applicant of the land subject of this permit must pay all of Council’s legal costs and expenses of this agreement, including preparation, execution and registration on title. - Section 173 Agreement – Publicly Accessible Land - Within 12 months of the date of issue of this permit, an agreement under Section 173 of the Planning and Environment Act 1987 must be entered into between the owner/s of the land and the Responsible Authority in a form satisfactory to the Responsible Authority to provide for: - a) Unrestricted public pedestrian, bicycle and vehicle access on all internal streets and thoroughfares including the piazza, approved under this permit, 24 hours a day, 364 days a year, commencing at the time of commencement of the use or the issue of a Statement of Compliance relevant to each Stage of this development, whichever comes first. - b) The owner/s of the land to indemnify Council against any claims associated with the use of the publicly accessible areas and to accept liability and responsibility for the on-going maintenance of the publicly accessible areas in perpetuity. - Prior to the development commencing, application must be made to the Registrar of Titles to Register the Section 173 Agreement on the title to the land under Section 181 of the Planning and Environment Act 1987. - The owner / permit applicant of the land subject of this permit must pay all of Council’s legal costs and expenses of this agreement, including preparation, execution and registration on title. - General conditions applying to Stages 1 and 2 - Secondary Consent - The use and development as shown on the endorsed plans must not be altered without the written consent of the Responsible Authority. - Environmental Auditing Requirements - Prior to the commencement of construction or carrying out of works for each relevant Stage approved under this permit, either: - A Certificate of Environmental Audit for the land must be issued in accordance with Section 53Y of the Environment Protection Act 1970 and provided to the Responsible Authority for the relevant Stage; or, - An Environmental Auditor appointed under Section 53S of the Environment Protection Act 1970 must make a Statement in accordance with Section 53Z of that Act that the environmental conditions of the land are suitable for the use and development that are the subject of the relevant Stage of this permit and that statement must be provided to the Responsible Authority. - Where a Statement of Environmental Audit is issued for the land, the buildings and works and the use(s) of the land that are the subject of this permit must comply with all directions and conditions contained within the Statement. - Where a Statement of Environmental Audit is issued for the land, prior to the commencement of the use, and prior to the issue of a Statement of Compliance under the Subdivision Act 1988, and prior to the issue of an Occupancy Permit under the Building Act 1993, a letter prepared by an Environmental Auditor appointed under Section 53S of the Environment Protection Act 1970 must be submitted to the Responsible Authority to verify that the directions and conditions contained within the Statement have been satisfied. Where a Statement of Environmental Audit is issued for the land, and any condition of that Statement requires any maintenance or monitoring of an ongoing nature, the Owner(s) must enter into an Agreement with Council pursuant to Section 173 of the Planning and Environment Act 1987. Where a Section 173 Agreement is required, the Agreement must be executed prior to the commencement of the permitted use, and prior to the certification of the plan of subdivision under the Subdivision Act 1988. All expenses involved in the drafting, negotiating, lodging, registering and execution of the Agreement, including those incurred by the Responsible Authority, must be met by the Owner(s). - Prior to any remediation works being undertaken in association with an Environmental Audit, a Remediation Works Plan, prepared in consultation with the appointed Environmental Auditor, must be submitted to and approved by the Responsible Authority. The plan must detail only those remediation works, excavation works as well as any proposed structures such as retaining walls, necessary to facilitate the completion of the environment audit. Only the works detailed in the Remediation Works Plan, approved by the Responsible Authority, are permitted to be carried out prior to the issue of a Certificate or Statement of Environmental Audit. - Environmental Sustainable Development - Prior to the endorsement of plans for each relevant Stage approved under this permit, the Sustainable Design in the Planning Process Plan prepared by Hampton Sustainability, Revision D dated 2 May 2014, must be amended by a suitably qualified environmental engineer or equivalent to the satisfaction of the Responsible Authority to address the following: - Stage 1 - a) A report relevant to Stage 1 only addressing best practice ESD at the time of lodgement. - b) Deletion of reference to bioswales on Nicholson Street. - Stage 2 - c) A report relevant to Stage 2 only addressing best practice ESD at the time of lodgement. - Stage 1 and 2 - d) A STORM model report that incorporates all impermeable areas demonstrating Best Practice stormwater management. e) Clear commitment to all initiatives listed as being considered in section 5.3 of the report with the following improvements: i. Reverse cycle air conditioning within 2 stars of best available (not less than 5 stars or equivalent). ii. Reverse cycle ducted split system with economy dampers to provide for heating and cooling of commercial areas. - f) Control mechanisms to ensure that the thermal chimneys will acceptably manage heat loss in winter. - g) Inconsistencies with the plans as detailed in Council’s ESD Engineer’s advice dated 7 March 2014, including but not limited to section 6 Water Resources. Where alternative ESD initiatives are proposed to those specified in condition 26d) – g), the Responsible Authority may vary the requirements of this condition at its discretion, subject to the development achieving equivalent (or greater) ESD outcomes in association with the development. When submitted and approved to the satisfaction of the Responsible Authority, the amended Sustainable Design in the Planning Process Plan and associated notated plans will be endorsed to form part of this permit. - All works must be undertaken in accordance with the endorsed Sustainable Design in the Planning Process Plan to the satisfaction of the Responsible Authority. No alterations to the Sustainable Design in the Planning Process Plan may occur without the written consent of the Responsible Authority. - Prior to the occupation of each relevant Stage approved under this permit, a report from the author of the Sustainable Design in the Planning Process Plan, approved pursuant to this permit, or similarly qualified person or company, must be submitted to the Responsible Authority. The report must be to the satisfaction of the Responsible Authority and must confirm that all measures specified in the Sustainable Design in the Planning Process Plan have been implemented in accordance with the approved Plan. - Accessibility - Prior to the endorsement of plans for each relevant Stage approved under this permit, an Access Plan must be prepared by a suitably qualified access auditor to assess any plans and provide advice / recommendations on access and mobility issues to the satisfaction of the Responsible Authority. The Plan must provide for, but not be limited to, the following: - Stage 1 - A report relevant to Stage 1 only. - A minimum of 29 adaptable dwellings or ten percent of the total number of dwellings, (whichever is greater), within the development (not optional). Inclusion of adaptable floor layouts in the Access Plan is required. - Stage 2 - A report relevant to Stage 2 only. - A minimum of 31 adaptable dwellings or ten percent of the total number of dwellings, (whichever is greater), within the development (not optional). Inclusion of adaptable floor layouts in the Access Plan is required. - Stage 1 and 2 - Vehicular and pedestrian access into the buildings. - Access to the lifts. - The provision of tactile indicators. - The provision of Braille indicators for the lifts. - The use of contrasting paving or surface materials to assist the vision impaired. - Emergency exits, including above the ground floor (e.g. evacuation chairs at stairs, or an alternative response to the satisfaction of the Responsible Authority). When submitted and approved to the satisfaction of the Responsible Authority, the Access Plan and associated notated plans will be endorsed to form part of this permit. - The recommendations of the plan must be implemented to the satisfaction of the Responsible Authority prior to the occupation of the development. No alterations to the Access Plan may occur without the written consent of the Responsible Authority. - Prior to the occupation of each relevant Stage approved under this permit, a report from the author of the Access Plan, approved pursuant to this permit, or similarly qualified person or company, must be submitted to the Responsible Authority. The report must be to the satisfaction of the Responsible Authority and must confirm that all measures specified in the Access Plan have been implemented in accordance with the approved Plan. - Waste Management - Prior to the endorsement of plans for each relevant Stage approved under this permit, the Waste Management Plan prepared by Leigh Design Pty Ltd dated 12 December 2013, must be amended by a suitably qualified consultant to the satisfaction of the Responsible Authority. The amended Plan must include, but not be limited to, the following: - Stage 1 - a) A report relevant to Stage 1 only. - Stage 2 - b) A report relevant to Stage 2 only. - Stage 1 and 2 - c) Dimensioned plans / diagrams at a scale of 1:100 or 1:200 showing the bin storage areas required in association with the development to accommodate the necessary bins, including circulation space. - When submitted and approved to the satisfaction of the Responsible Authority, the Waste Management Plan will be endorsed to form part of this permit. - All waste / recycling areas and stores are to be provided with ventilation in accordance with Australian Standard AS1668. - The endorsed Waste Management Plan must be implemented and complied with at all times to the satisfaction of the Responsible Authority. The Waste Management Plan can be implemented by any waste company that complies with the waste management plan to the satisfaction of the Responsible Authority. No alterations to the Waste Management Plan may occur without the written consent of the Responsible Authority. - Landscaping - Prior to the endorsement of plans for each relevant Stage approved under this permit, the Concept Landscape Plan prepared by Habitat dated March 2014, must be amended by a suitably qualified consultant to the satisfaction of the Responsible Authority. The amended Plan must include, but not be limited to, the following: - Stage 1 - a) A plan relevant to Stage 1 only. - b) Details of the first floor communal landscaped areas to Lots 1, 2 and 3. - c) Details of the planter box planting to the perimeter of Lot 4. - d) Deletion of the raised planter boxes along the Nicholson Street frontage and replacement with standard tree cut-outs. - Stage 2 - e) A plan relevant to Stage 2 only. - f) Details of the first floor communal landscaped areas to Lots 4 and 7. - g) Details of the roof garden to Lot 5. - h) Details of the rain gardens / bioswales. - i) Deletion of the northern-most planter box to the east of Lot 7. - Stage 1 and 2 - j) A schedule of all proposed planting (including numbers, size at planting, size at maturity and botanical names) within the common areas at ground, first floor and roof level. The flora selection and landscape design should be drought tolerant and must respond appropriately to localised site conditions. - k) Species amended to reflect the requirements of Council’s Open Space Design and Development Unit’s advice dated 13 May 2014. - l) Details of raised planter beds and boxes, including height, construction methods, maintenance and location to alleviate any sight line issues. - m) Integration of water sensitive urban design (WSUD) features, including investigation of opportunities for passive irrigation (kerb cut outs). - n) Details of paving. - o) Location and design of bins, seating, lighting and any landscape features. - p) Lighting within publicly accessible areas. - q) Advice detailing how the landscaping will be constructed and accommodated, including areas required for planting, drainage and irrigation. - r) Delineation of all public and private areas. - s) A maintenance schedule. When submitted and approved to the satisfaction of the Responsible Authority, the Landscape Plan will be endorsed to form part of this permit. All works must be undertaken in accordance with the endorsed Landscape Plan to the satisfaction of the Responsible Authority. No alterations to the Landscape Plan may occur without the written consent of the Responsible Authority. - Prior to the occupation of each relevant Stage approved under this permit, the areas designated for landscaping on the endorsed plan must be planted to the satisfaction of the Responsible Authority. - Following completion of the development, the areas designated for landscaping on the endorsed plan must be maintained to the satisfaction of the Responsible Authority, including that any dead, diseased or damaged plants are to be replaced and any publicly accessible areas repaired and maintained. Public Lighting - Prior to occupation of each relevant Stage or prior to the issue of a Statement of Compliance for each relevant stage of the subdivision, whichever comes first, a report detailing that all public lighting installed achieves compliance with the Australian Standard for lighting AS1158 ‘Lighting for Roads and Public Spaces’ to Category P4 must be provided to the satisfaction of the Responsible Authority. Commercial Noise Restrictions - The maximum noise level emitted from the commercial premises must not exceed levels specified in the State Environment Protection Policy (Control of Noise from Commercial, Industrial or Trade Premises within the Melbourne Metropolitan Area), SEPP N-1. - Traffic and Parking - Prior to the occupation of the development, ‘No Stopping’ (at all times) signs must be erected and maintained in locations necessary to ensure that Metropolitan Fire Brigade (MFB) fire truck emergency access is maintained at no cost to Council and to the satisfaction of the Responsible Authority. - The car parking spaces provided on the land must be solely associated with the development allowed by this permit and must not be subdivided or sold separately from the development for any reason without the written consent of the Responsible Authority to the satisfaction of the Responsible Authority. - The roller shutter between the east and west residential basement car parks must only be accessible to cars that park in the western car park. - All vehicle entry roller shutters / doors are to be secured with semi-transparent security grilles, or an alternative design to the satisfaction of the Responsible Authority, (entries to loading bays excepted). - The internal roads, car parks and loading bays must be designed in accordance with the standards approved in the Traffic and Transport Assessment prepared by Cardno Victoria Pty Ltd dated 27 March 2013. - Prior to the occupation of the development, any Council or service authority pole or pit within 1 metre of a proposed vehicle crossing must be relocated or modified at the expense of the permit holder to the satisfaction of the Responsible Authority and the relevant service authority. - Prior to the occupation of the development, a vehicle crossing must be constructed in every location shown on the endorsed plans to a standard satisfactory to the Responsible Authority (Moreland City Council, City Infrastructure Department). - Prior to the occupation of the development, any existing vehicle crossing not to be used in this development must be removed and the kerb and channel and footpath reinstated to the satisfaction of the Responsible Authority (Moreland City Council, City Infrastructure Department). - Prior to the occupation of the development, the bicycle parking racks must be installed in a secure manner that accords with the specifications in Bicycle Victoria’s Bicycle Parking Handbook, to the satisfaction of the Responsible Authority. - Prior to the occupation of the development, bicycle signage at least 300mm wide and 450mm high showing a white bicycle on a blue background must be erected directing cyclists to the location of the bicycle parking as required by Clause 52.34-5 of the Moreland Planning Scheme. - Loading / Unloading - Unless otherwise agreed by the Responsible Authority in writing, all truck deliveries in association with the use of the completed development, other than any supermarket, are restricted to: - a) 7am to 10pm Monday to Saturday; and - b) 9am to 10pm Sunday and public holidays. - Drainage - All stormwater from the land, where it is not collected in rainwater tanks for re-use or managed within rain gardens / bioswales, must be collected by an underground pipe drain approved by and to the satisfaction of the Responsible Authority (Moreland City Council, City Infrastructure Department). - The rain gardens / bioswales are to be regularly maintained by the owner in accordance with the requirements in the Melbourne Water WSUD Engineering Procedures: Stormwater guidelines, to the satisfaction of the Responsible Authority. - General - Prior to the issuing of Statement of Compliance or occupation of the development, whichever occurs first, all visual screening measures shown on the endorsed plans must be installed to the satisfaction of the Responsible Authority. All visual screening and measures to prevent overlooking must be maintained to the satisfaction of the Responsible Authority. Any screening measure that is removed or unsatisfactorily maintained must be replaced to the satisfaction of the Responsible Authority. - Prior to the occupation of the development, all boundary walls must be constructed, cleaned and finished to the satisfaction of the Responsible Authority. - Unless with the prior written consent of the Responsible Authority, any plumbing pipe, ducting and plant equipment must be concealed from external views to the satisfaction of the Responsible Authority. This does not include external guttering or associated rainwater down pipes. - Prior to the occupation of the development, air-conditioning and other plant and generator equipment must be screened from the view of adjoining properties, streets and all other publicly accessible areas to the satisfaction of the Responsible Authority. - Prior to the occupation of the development, all telecommunications and power connections (where by means of a cable) and associated infrastructure to the land must be underground to the satisfaction of the Responsible Authority. - A letterbox must be provided for each of the premises. The dimensions, placement and numbering must comply with the Australia Post – Letterbox Security and Specification as published on its website to the satisfaction of the Responsible Authority. - Public Transport Victoria conditions - The permit holder must take all reasonable steps to ensure that disruption to tram operations along Nicholson Street is kept to a minimum during the construction of the development. Foreseen disruptions to tram operations during construction and mitigation measures must be communicated to Yarra Trams and Public Transport Victoria fourteen days (14) prior. - The permit holder must take all reasonable steps to ensure that disruption to bus operations and bus stops along Nicholson Street is kept to a minimum during the construction of the development. Forseen disruptions to bus operations and mitigation measures must be communicated to Public Transport Victoria fourteen days (14) prior. - The permit holder must ensure that public transport infrastructure is not altered without the consent of Public Transport Victoria or damaged. Any damage to public transport infrastructure must be rectified to the satisfaction of Public Transport Victoria at the full cost of the permit holder. - VicRoads conditions - Before the development starts, amended plans must be submitted to and approved by VicRoads and the Responsible Authority. When approved by VicRoads, the plans may be endorsed by the Responsible Authority and will then form part of the permit. The plans must be drawn to scale with dimensions and two copies must be provided. The plans must be generally in accordance with the plan (Cardno, Drw. No. CG111076 T 01, Rev.P, Dated 15/03/11) which was submitted with the Development Plan, but modified to show: - a) Proposed signals at the intersection of Nicholson Street / Sumner Street / Proposed access point including tram priority requirement for interim and ultimate stages in accordance with AustRoads’ “Guide to Road Design 2010” and relevant Victorian supplements; - b) Proposed signage and line marking in accordance with the VicRoads Traffic Engineering Manual – Volume 2 Signage and Markings (January 2010). - Prior to the commencement of any works at the intersection of Nicholson Street / Sumner Street / Proposed access point, functional layout plans must be submitted to VicRoads and the Responsible Authority for approval for the following works: - a) Proposed signals at the intersection of Nicholson Street / Sumner Street / Proposed access point including tram priority requirement for interim and ultimate stages in accordance with AustRoads’ “Guide to Road Design 2010” and relevant Victorian supplements; - b) Proposed signage and line marking in accordance with the VicRoads Traffic Engineering Manual – Volume 2 Signage and Markings (January 2010). - Before the use approved by this permit commences, the road works approved as per Condition No.62 for the interim stage and any other associated work must be completed at no cost to and to the satisfaction of VicRoads and the Responsible Authority. - Expiry - This permit will expire if one of the following circumstances applies: - Stage 1 - Stage 1 of the development is not commenced within three (3) years from the date of issue of this permit; - Stage 1 of the development is not completed within five (5) years from the date of issue of this permit; - the use of Stage 1 is not commenced within five (5) years from the date of issue of this permit. Stage 2 - Stage 2 of the development is not commenced within five (5) years from the date of issue of this permit; - Stage 2 of the development is not completed within seven (7) years from the date of issue of this permit; - the use of Stage 2 is not commenced within seven (7) years from the date of issue of this permit. The Responsible Authority may extend the periods referred to if a request is made in writing before the permit expires or; - Within six months after the permit expires to extend the commencement date. - Within 12 months after the permit expires to extend the completion date of the development if the development has lawfully commenced. - NOTES: - These notes are for information only and do not constitute part of this Permit. - Note 1: Should Council impose car parking restrictions in this street, the owners and/or occupiers of the land would not be eligible for any Council parking permits to allow for on street parking. - Note 2: i. A copy of the Certificate or Statement of Environmental Audit, including the complete Environmental Audit Report must be submitted to the Responsible Authority within 7 days of issue, in accordance with Section 53ZB of the Environment Protection Act 1970. ii. Where a Statement of Environmental Audit is issued for the land a copy of that Statement must be provided to any person who proposes to become an occupier of the land, pursuant to Section 53ZE of the Environment Protection Act 1970. iii The land owner and all its successors in title or transferees must, upon release for private sale of any part of the land, include in the Vendor’s Statement pursuant to Section 32 of the Sale of Land Act 1962, a copy of the Certificate or Statement of Environmental Audit including a copy of any cover letter. iv. Where a Statement of Environmental Audit issued for the land contains conditions that the Responsible Authority considers to be unreasonable in the circumstances, the Responsible Authority may seek cancellation or amendment of the planning permit in accordance with Section 87 of the Planning and Environment Act 1987. - Note 3: Council’s public lighting policy mandates that energy efficient technologies be employed (i.e. LED or T5 fluorescent) and luminaires be mounted at minimum 6 metre mounting height. - Note 4: Council may install a prohibition on parking on John Street at the western end of ‘Main Street’ to facilitate emergency vehicle access. Note 5: Council charges supervision (2.50%) and plan checking (0.75%) fees on the cost of constructing the drain along the easement or street as permitted by Sections 5 & 6 of the Subdivision (Permit and Certification Fees) Regulations 2000. Note 6: Further approvals are required from Council’s City Infrastructure Department who can be contacted on 9240 1143 for any works beyond the boundaries of the property. Planting and other vegetative works proposed on road reserves can be discussed with Council’s Open Space Unit on 8311 4300. Note 7: This permit does not authorise the removal of, damage to or pruning of any street tree. Further written consent must be obtained from Council’s Open Space Unit. The Open Space Unit can be contacted on 8311 4300. - Note 8: Unless no permit is required under the Moreland Planning Scheme, no sign must be constructed or displayed on the land without a further planning permit.

Do not delete this line

REPORT

## 1.	Background

### Subject Site

The subject site is located on the west side of Nicholson Street, midblock between Albert Street to the north and Glenlyon Road to the south. The site has a 101 metre wide frontage to Nicholson Street and a 92 metre wide frontage to John Street. The site includes rear access from Elm Grove, Gamble Street and Rickard Street.

The site has an area of approximately 21,300 square metres (2.13 hectares/5.2 acres) and an east-west depth of approximately 200 metres.

The majority of the site contains industrial buildings previously used by the bedding manufacturers ‘Tontine’. These buildings, and a two storey office building on Nicholson Street previously occupied by the Australian Institute of Tourism and Commerce, are now vacant.

A two storey building in the north east corner of the site is occupied by the Victorian Aboriginal Child Care Agency (VACCA).

The subject site includes a small part of 149 Nicholson Street which is occupied by the South Pacific Laundry. The part of the laundry site which is proposed to be developed under this application does not include the large rectangular factory used by the South Pacific Laundry.

There are no restrictive covenants indicated on the relevant Certificates of Title.

### Surrounds

The surrounding area is characterised by a mix of industrial, commercial and residential uses.

Heavy industry exists adjacent to the north of the site at 96-100 John Street and 2 Elm Grove – the Townley Steel Fabrication and Drop Forge factories – and 149 Nicholson Street – the South Pacific Laundry.

- The South Pacific Laundry site has recently been purchased by the developer who has advised that the commercial laundry use will be vacating in the coming months.

A mix of light industries, a motor repairs use and commercial uses exist south of the site on Nicholson Street, Rickard Street and west of the site on John Street. An artist’s studio exists at the rear of 80 John Street.

Housing ranging in scale from 1 to 3 storeys exists on the opposite side of Nicholson Street, and on Glenlyon Road, Gamble, John and Albert Streets.

Planning Permit MPS/2013/1022 was issued on 9 May 2014 for the development of a 5 storey building containing 44 dwellings and 2 retail premises (with frontage to Rickard Street) at 6-8 Gamble Street, adjacent to the subject site. The development has not commenced.

Planning Permit MPS/2009/424 was issued on 9 November 2010 for the development of a 5 storey building containing 150 dwellings, offices and a food and drink premises at 48-58 Albert Street, diagonally opposite the subject site. The development is under construction.

The site is serviced by the Nicholson Street tram route 96 and bus routes 506 (Glenlyon Road) and 508 (Blyth Street). John Street is on the north-south bicycle route known as the East Brunswick Shimmy. Flexi cars are stationed in Glenlyon Road and Blyth Street.

A location plan forms Attachment 1.

### The Proposal

The application proposes the development and use of the land for seven buildings comprising three 5 storey buildings and four 6 storey buildings within a new network of streets and pedestrian / cycle connections over 2 basement levels.

Details include:

- 603 dwellings (241 x 1 bedroom; 360 x 2 bedroom; 2 x 3 bedroom)

- 24 hour supermarket with loading bay access from Rickard Street

- 17 shops with loading bay access from Rickard Street (Lot 1) and Nicholson Street (Lots 3 and 4)

- 3 food and drink premises

- 5 home offices

- 748 car parking spaces (525 residential; 223 retail)

- 752 bicycle parking spaces (614 residential; 48 retail; 90 visitor)

- 3 bicycle commuter showers for retail staff use

- 615 x 4 m3 storage units (603 residential; 12 retail)

Stage 1:

- Development of Basements 1 and 2, and Lots 1, 2 and 3

- Public realm works including:

- ‘Main Street’ (excluding plaza / mall) and the ‘Main Street’ / Nicholson Street signalised intersection

- Internal roads between Lots 3 and 4 and Lots 4 and 6, with ramp access to basement retail and residential car parking

- Pedestrian and cycle connections between Lots 1 and 2 and Lots 2 and 5 to Rickard and Gamble Streets respectively

- 292 dwellings (138 x 1 bedroom; 154 x 2 bedroom)

- 24 hour supermarket

- 12 shops

- 1 home office

- Vehicle access to 526 car parking spaces (303 residential; 223 retail).

- Bicycle access to 614 bicycle parking spaces (530 residential; 48 retail; 36 visitor) and 3 bicycle commuter showers

- 460 x 4 m3 storage units (448 residential; 12 retail)

Stage 2:

- Development of Lots 4, 5, 6 and 7

- Public realm works including:

- Pedestrian plaza / mall and piazza, (western end of the new ‘Main Street’ to John Street)

- Pedestrian and cycle connections between Lots 6 and 7 to Elm Grove

- Streetscape improvement works to the John Street frontage

- Intersection thresholds installed at Glenlyon Road / John Street and Albert Street / John Street, and a road hump in John Street

- 311 dwellings (103 x 1 bedroom; 206 x 2 bedroom; 2 x 3 bedroom)

- 5 shops

- 4 home offices

- Vehicle access to 222 car parking spaces (residential)

- Bicycle access to 138 bicycle parking spaces (84 residential; 54 visitor)

- 155 x 4 m3 storage units (residential)

The application plans for Stage 1 and 2 are in the Revision C set dated 22 May 2014. The Revision D set of plans dated 26 November 2014 are identical to the Revision C set in all respects except that they only show the development proposed as part of Stage 1.

No sale or consumption of liquor is proposed under this application.

The development plans (excerpts only) including plans, elevations, sections, a landscaping plan and staging plan form Attachments 2 to 6.

### Planning Amendment History

Amendment C92 to the Moreland Planning Scheme came into operation on 20 January 2011 to facilitate the creation of a new Neighbourhood Activity Centre on the land generally bordered by Nicholson Street, Glenlyon Road, John Street and Albert Street, Brunswick East, (including the subject site), described as ‘the Precinct’.

Under Amendment C92, industrially zoned land was rezoned to Business 1 and 2 Zone (now Commercial 1 Zone) to enable consideration of a mix of retail, commercial, residential and community uses within the precinct, consistent with the Brunswick Structure Plan. The Amendment also applied an Environmental Audit Overlay to the majority of the land.

As part of Amendment C92, Schedule 11 to the Development Plan Overlay (DPO11) was introduced to the Moreland Planning Scheme. DPO11 details requirements for the preparation of Development Plan(s) to the satisfaction of the Responsible Authority to guide the future development of the precinct.

East Brunswick Village Development Plan

Under Clause 43.04-1 of the Development Plan Overlay, a permit must not be granted to use land, construct a building or construct or carry out works until a development plan has been prepared to the satisfaction of the Responsible Authority.

The East Brunswick Village (EBV) Development Plan was endorsed by Council on 4 October 2012 as satisfying the DPO11 requirements for the preparation of a Development Plan for part of the precinct, including the subject site. The endorsed EBV Development Plan prescribes building envelopes, requirements for public realm works, vehicle and pedestrian circulation, ESD expectations and developer contributions. It guides the future development of 31,221 square metres, (63 per cent) of the precinct, including the precinct’s largest land parcels.

The subject planning application relates to 21,304 square metres (68 per cent) of the endorsed EBV Development Plan area.

Endorsement of the EBV Development Plan occurred after public consultation, Council consideration and a review by the Victorian Civil and Administrative Tribunal. The EBV Development Plan, including all endorsed documents, can be downloaded from Council’s website: http://www.moreland.vic.gov.au/building-and-planning/strategic-projects-moreland/east-brunswick-development-plan.html

Relevant specific aspects of the endorsed EBV Development Plan as relevant to the subject planning application include:

- Building envelopes to 6 storeys, incorporating 3 and 4 storey street walls to Nicholson Street and John Street respectively.

- Public realm works and access:

- Construction of a new east-west ‘Main Street’ opposite Sumner Street, including a signalised intersection at Nicholson Street.

- Inclusion of an ’urban piazza’ within ‘Main Street’.

- Construction of internal streets and thoroughfares between buildings with unrestricted public pedestrian and cycle access. Private ownership and maintenance of publicly accessible areas by an owners’ corporation.

- Indicative land use and car spaces:

- 550-650 dwellings.

- 3000 sq m supermarket.

- 4000 sq m specialty shops.

- 855 car spaces (562 residential; 293 retail).

- Developer contributions:

- Signalised intersection at Nicholson Street, including tram priority and pedestrian crossing(s).

- One road hump in John Street.

- Intersection thresholds installed at Glenlyon Road / John Street and Albert Street / John Street.

- Community infrastructure levy in accordance with Amendment C133 (as exhibited), to be facilitated by a Section 173 Agreement.

The endorsed EBV Development Plan building envelope forms Attachment 7.

### Statutory Controls – why is a planning permit required?

| Control | Permit Requirement |
| --- | --- |
| Commercial 1 Zone | The application includes the following uses: - Dwellings - Supermarket - Shops - Food and drink premises - Offices Of these, supermarket, shop, food and drink premises and offices are Section 1 uses in the zone, meaning that a permit is not required for these uses under Clause 34.01-1. Clause 34.01-1 – ‘Dwelling’ (where the frontage at ground level exceeds 2 metres) is a Section 2 use in the zone, meaning that a permit is required for this use as all frontages (including entrance lobbies) to dwellings at ground level exceed 2 metres. Clause 34.01-4 – A permit is required to construct a building or construct or carry out works. |
| Car Parking | Clause 52.06-3 – A permit is required to reduce the car parking requirement by 205 spaces (from 953 spaces to 748 spaces). The reduced spaces comprise 80 resident spaces, 120 residential visitor spaces and 16 office spaces. The 223 car parking spaces provided for the retail uses (supermarket, shop and food and drink premises) exceed the requirement by 11 spaces. |
| Land Adjacent to a Road Zone, Category 1 | Clause 52.29 – A permit is required to create access to a road in a Road Zone, Category 1. |

While not permit triggers, the Development Plan Overlay (DPO11) and the Environmental Audit Overlay (EAO) are relevant to the consideration of this proposal.

The following Particular Provisions of the Moreland Planning Scheme are also relevant to the consideration of the proposal:

- Clause 52.07: Loading and Unloading of Vehicles. No permit is required as the required provision is met.

- Clause 52.34: Bicycle facilities. No permit is required as the required provision is met, including employee showers and communal change room.

- Clause 52.35: Urban context report and design response for residential development of five or more storeys

- Clause 52.36: Integrated public transport planning

## 2.	Internal/External Consultation

### Public Notification

Under Clause 43.04-2 of the Moreland Planning Scheme, the application is exempt from public notice as it is generally in accordance with the EBV Development Plan endorsed by Council on 4 October 2012.

### External Referrals

| External Agency | Objection/No objection |
| --- | --- |
| VicRoads | No objection subject to conditions including a requirement to provide a signalised intersection at Nicholson Street, incorporating tram priority, at no cost to VicRoads or Council. |
| Public Transport Victoria | No objection subject to conditions relating to minimisation of disruption to tram and bus services and rectification of any damages to public transport infrastructure. |
| Environmental Protection Authority | The EPA is not a statutory referral authority to this matter. A copy of the application was sent to them for consideration and comment in respect to the proposed location of dwellings directly adjacent to heavy industries. The EPA has advised that they are not supportive of the application which proposes dwellings adjacent to the South Pacific Laundry and Townley Drop Forge as these uses are incompatible. Approval of the scheme would result in a high potential for adverse noise and vibration impacts affecting new residents, and would severely disadvantage existing industry. This issue is addressed in the recommendation which precludes endorsement of plans and the construction of any dwellings on Lots 4, 6 and 7 unless the Responsible Authority is satisfied that the adverse amenity impacts of noise and vibration from the remaining adjacent heavy industry has been resolved or satisfactorily addressed. |

Internal Referrals

| Internal Department | Comments |
| --- | --- |
| City Strategy and Design Branch | No objection subject to conditions. The public spaces are well resolved with a high level of pedestrian and cycling amenity. Pedestrian amenity and the public realm could be improved by: - Locating a threshold / raised crossing across the street between Lots 3 and 4, generally in keeping with the threshold treatment between Lots 4 and 6. |
| City Strategy and Design Branch (cont.) | - Exploring opportunities to improve visual interest in ‘Main Street’ through detailed design of the building facades and material finishes. - Better utilising the northern aspect in Lot 1. - Use of transparent / semi-transparent materials at ground floor to the stairwells in Lot 2. These matters are addressed in conditions in the recommendation. |
| Strategic Transport and Urban Safety Branch | No objection subject to conditions and notes to address road safety, emergency vehicle access, loading, waste management, traffic, parking and drainage. |
| ESD Unit | Subject to conditions requiring clarification and clear commitments to deliver the best practice initiatives proposed, the application meets a best practice ESD standard equivalent to 4 Star Green Star as required for a development of this scale. |
| Open Space Design and Development Unit | No objection subject to detailed designs, replacement of planter boxes along Nicholson Street with standard tree cut-outs, modified species to reflect local conditions and integration of water sensitive urban design (WSUD) features. |

## 3.	Policy Implications

### State Planning Policy Framework (SPPF)

The following State Planning Policies are of most relevance to this application:

- Clause 9: Plan Melbourne

- Clause 11.01: Activity Centres

- Clause 11.02: Urban Growth

- Clause 11.04: Metropolitan Melbourne

- Clause 13.04: Noise and Air

- Clause 15.01: Urban Environment

- Clause 15.02: Sustainable Development

- Clause 16.01: Residential Development

- Clause 17.01: Commercial

- Clause 18.01: Integrated Transport

- Clause 18.02: Movement Networks

### Local Planning Policy Framework (LPPF)

The following Strategic Directions of the Municipal Strategic Statement (MSS) and the following Local Planning Policies are of most relevance to this application:

Municipal Strategic Statement:

- Clause 21.03-1 Activity Centres

- Clause 21.03-3 Housing

- Clause 21.03-4 Urban Design, Built Form and Landscape Design

- Clause 21.03-5 Environmentally Sustainable Design (Water, Waste and Energy)

- Clause 21.03-6 Open Space Network

Local Planning Policies:

- Clause 22.03 Car and Bike Parking and Vehicle Access

- Clause 22.07 Development of Four or More Storeys

### Planning Scheme Amendments

#### Amendment C134 – Brunswick Structure Plan

Amendment C134 seeks to implement the recommendations of the Brunswick Structure Plan (2010) and the Addendum to the Brunswick Structure Plan (2012) as permanent planning controls within the Moreland Planning Scheme.

Under Amendment C134, the land is affected by DDO20 ‘Brunswick Major Activity Centre – Nicholson Street Local Area’. However, part 2.0 of the proposed DDO20 states that:

‘A permit is not required if the subject site forms part of a development plan approved in accordance with Schedule 11 to the Development Plan Overlay (DPO11)’.

#### Amendment C142 – Moreland Apartment Design Code

Amendment C142 seeks to introduce the Moreland Apartment Design Code (MADC) as an incorporated document under the Moreland Planning Scheme.

Public exhibition of the recently revised Amendment concluded on 3 October 2014.

The MADC is pending further consultation and analysis of a peer review prior to consideration by an independent Panel, scheduled to commence on 23 March 2015.

#### Amendment C71 – Environmentally Efficient Design Policy

Amendment C71 seeks to introduce a new local policy ‘Environmentally Sustainable Development’. The Amendment is pending Ministerial approval.

#### Amendment C133 – Development Contributions Plan Overlay

Amendment C133 proposes to implement the Moreland Development Contributions Plan 2014.

Under Amendment C133, Schedule 1 to the Development Contribution Plan Overlay (DCPO1) will apply to land, imposing development infrastructure and community infrastructure levies to fund a range of social and physical local infrastructure in accordance with the Moreland Development Contributions Plan 2014.

The independent Panel report on the Amendment was reported to Council at its February 2015 meeting.

### Human Rights Consideration

This application has been assessed in accordance with the requirements of the Planning and Environment Act 1987 (including the Moreland Planning Scheme), reviewed by the State Government and which complies with the Victorian Charter of Human Rights and Responsibilities Act 2006.

## 4.	Issues

In considering this application, regard has been given to the State and Local Planning Policy frameworks, the provisions and proposed amendments to the Moreland Planning Scheme, the approved East Brunswick Village (EBV) Development Plan and the individual merits of the application.

### Adverse amenity impacts

The main issue of relevance to this application is whether approval should be granted to allow for the construction and use of dwellings on Lots 4, 6 and 7 as part of Stage 2 of the development.

An expert adverse amenity report has been submitted by the applicant. The report does not consider adverse impacts (noise, dust, odour, etc.) on the proposed use from the South Pacific Laundry. This is accepted as the laundry is due to vacate in the coming months, subject to the cancellation of the planning permit which currently allows the use to operate.

The expert adverse amenity report indicates that dwellings on Lots 4, 6 and 7 cannot be acoustically treated on-site to protect them from significant noise emanating from the Townley Drop Forge without the need for a complete redesign of the buildings, or the construction of a solid 17 metre high screen wall (acoustic barrier) along the northern perimeter of the subject site. Either approach is not supported, noting that: a complete redesign of the buildings would need to be considered under a new planning application or a formal amendment to the existing application; and the erection of a 17 metre high acoustic wall would have an unacceptable visual impact on the public realm and the internal amenity of the affected dwellings.

The Townley Drop Forge is located at 2 Elm Grove, directly north of proposed Lot 6. It operates between 8am and 4pm, Monday to Friday and forges bolts, eye nuts, hooks and other metal fasteners and lifting points. The Forge operates an electrically heated furnace, two drop hammers and a shot blasting drum. The Effective Noise Level emanating from the Forge is 7 dBA above the EPA day time noise limit for residential uses proposed in Lots 4, 6 and 7.

The recommendation is for a permit to issue which allows for the construction and use of dwellings on Lots 4, 6 and 7 as part of Stage 2 of the development subject to:

- Endorsement of plans for Stage 2 of the development being withheld until expert advice to Council’s satisfaction is received confirming that occupation of the dwellings in Lots 4, 6 and 7 would not result in a breach of the State Environment Protection Policy No. N-1 (SEPP N-1)*, without:

- The need to construct a high acoustic wall.

- Design modifications which would result in unreasonable impacts on internal or external amenity (e.g. in terms of ventilation, outlook and bulk).

- The development ceasing to be generally in accordance with the endorsed East Brunswick Village (EBV) Development Plan.

*	SEPP N-1 is the tool used to protect residents in metropolitan Melbourne from excessive noise from industry.

Without endorsed plans under the planning permit, no building permit could issue for the Stage 2 works. This would prevent the dwellings in Lots 4, 6 and 7 from being constructed before the adverse amenity non-compliances associated with the Townley Drop Forge are satisfactorily addressed.

- The owner of the land entering into a legal agreement with Council prior to the commencement of Stage 1 works, agreeing to:

- Not subdivide the Stage 2 land until plans are first endorsed for Stage 2 (demonstrating that SEPP N-1 will not be breached).

- Endorsement of plans for Stage 2 being withheld until the adverse amenity issues are resolved.

- Provide evidence of completion of any off-site mitigation undertaken ‘at the source’ to address adverse amenity impacts prior to the endorsement of plans.

The requirement for the owner to enter into such an agreement with Council, (which is required to be registered on the title to the land, and which remains registered on title with any subsequent change of ownership), ensures that any prospective purchaser of the land is made clearly aware of the constraints to developing Lots 4, 6 and 7.

The restriction on subdividing the Stage 2 land before Stage 2 plans are first endorsed will ensure that parcels are not on-sold which may not be able to be developed if the Townley Drop Forge continues to operate in its current form. It will also facilitate the integrated delivery of the public realm works included as part of Stage 2.

The recommendation to issue a permit subject to conditions is considered preferable to refusing the application for the following reasons:

- With the exception of the adverse amenity constraints to the development of residential uses as part of Stage 2, the overall scheme is highly resolved and worthy of approval.

- It is possible that the Townley Drop Forge may relocate elsewhere in the medium-term. The relocation of the Forge would resolve the adverse amenity issues, but cannot be relied upon to happen. The owner / developer of the subject development site is willing to take this risk however, noting that the construction of Stage 2 would only be ready to commence in approximately five years.

- The permit conditions provide the developer with the option to attenuate the Forge ‘at the source’, prior to the endorsement of plans for Stage 2. This could provide a solution, albeit costly to the developer, potentially allowing the Forge and residential uses under Stage 2 to coexist.

The expert adverse amenity report details:

- Ground vibration impacts on Lots 4, 6 and 7 from the Forge would be excessive, but can be addressed through mitigation treatment to isolate the transfer of vibration into the buildings.

- Lots 1, 2, 3 and 5 are not unreasonably adversely affected by noise or vibration from the Forge.

- Other nearby industries, including the Townley Steel Fabrication and motor vehicle repair businesses, emit little or no environmental noise and no vibration requiring specific attenuation to the proposed development.

- Odour impacts from nearby industries are considered negligible requiring no response.

Noise, dust and odour impacts on the proposed dwellings from the adjacent South Pacific Laundry at 149 Nicholson Street have been resolved with the recent purchase of this site by the developer who has advised that the business intends to relocate.

### Does planning strategy support the use and development in this location?

The relevant zone and overlay provisions under the Moreland Planning Scheme have been introduced to facilitate the development and use of a mix of activities on and adjacent to the subject site, generally in accordance with the endorsed East Brunswick Village (EBV) Development Plan.

Therefore, policy support exists at both State and Local level for a mixed use development with increased housing intensity on this site as proposed.

The subject land is located within the Brunswick Major Activity Centre, with good access to public transport, cycling networks, infrastructure and services.

### Developer contributions / community benefit

#### Signalised intersection at Nicholson Street / ‘Main Street’

Under the East Brunswick Village (EBV) Development Plan, a signalised intersection at Nicholson Street and the new ‘Main Street’, including tram priority and pedestrian crossings, is required to service the development. The EBV Development Plan details an ‘interim’ intersection treatment and road reserve which does not accommodate a fully accessible DDA compliant tram platform, and an ‘ultimate’ (larger) intersection and road reserve which could accommodate a DDA compliant tram platform / ‘super stop’.

Under the EBV Development Plan, the developer is required to construct and cover the cost of the signalised intersection, including tram priority and pedestrian crossings, in accordance with the interim layout only. While the developer is not required to construct and cover the cost of the ultimate intersection, it is required to setback buildings from Nicholson Street so that they do not encroach on the ultimate road reserve.

The Victorian Civil and Administrative Tribunal (VCAT) confirmed that the cost of the interim intersection treatment only, was to be met by the developer in its decision of East Brunswick Village Pty Ltd v Moreland CC [2012] VCAT 1307 dated 3 September 2012 at paragraph 76:

EBV will bear the cost of the interim intersection and the authorities have agreed to bear the costs of the DDA compliant tram stop.

In its decision, VCAT noted that the DDA compliant tram stop was part of a larger public benefit and that the nexus between the EBV Development Plan and the intersection is limited to the interim works and the ‘development loss’ of setting buildings back behind the ultimate alignment.

Further to this, the ultimate intersection cannot be constructed without VicRoads first acquiring or negotiating the transfer of land from adjacent private titles at 115, 119-121, 123, 125 and 149 Nicholson Street.

The application plans indicate that the proposed development has been setback behind the ultimate road reserve boundary. The recommendation requires amended plans to clearly show the interim and ultimate road reserve boundaries on all floor plans, and further requires the owner to construct the interim signalised intersection at no cost to Council.

#### John Street road works

Under the EBV Development Plan, traffic calming works on John Street are required to be delivered as part of the development to minimise amenity impacts on existing residents from the increased traffic volumes resulting directly from the development.

VCAT, in its decision of East Brunswick Village Pty Ltd v Moreland CC [2012] VCAT 1307 dated 3 September 2012, stated at paragraph 77:

The additional traffic volumes on John Street are a direct result of this [Development Plan]. ... Accordingly, additional works on John Street are reasonable, given they result directly and solely from this [Development Plan].

In its Order dated 3 September 2012, VCAT required the EBV Development Plan to be amended to insert:

One road hump will be installed in John Street to the satisfaction of the Council. Intersection thresholds will be installed at Glenlyon Road / John Street and Albert Street / John Street to the satisfaction of the Council.

The recommendation requires the applicant to deliver the John Street traffic calming works prior to the subdivision of Stage 2, or within five years of the date of issue of the permit, whichever comes first, at no cost to Council.

#### John Street streetcape improvements

While not specifically required under the EBV Development Plan, the developer is accepting the responsibility to provide streetscape improvement works along the John Street frontage. The recommendation requires a public works plan to be submitted to Council’s satisfaction including consideration of the planting of street trees, footpath works, relocation / replacement of services and street furniture. Implementation of the approved plan is required to be undertaken prior to the occupation of Stage 2 of the development at no cost to Council.

#### Community infrastructure contribution

Schedule 11 to the Development Plan Overlay (DPO11) foreshadowed the need for a community infrastructure developer contribution (monetary or building) to be addressed under the Development Plan. The endorsed EBV Development Plan requires the developer to enter into a Section 173 Agreement with Council to pay a community infrastructure cash contribution based on the Moreland Development Contributions Plan. The developer is accepting of this requirement which is reflected in the recommendation, requiring payment of the cash contribution prior to the issue of a Building Permit for each Stage of the development.

#### Internal streets, ‘Main Street’, and the piazza

Schedule 11 to the Development Plan Overlay (DPO11) foreshadowed the need for a safe shared vehicle and pedestrian environment within ‘Main Street’, connecting Nicholson Street and John Street as the focus for the precinct. DPO11 also required connections from Rickard Street, Gamble Street and Elm Grove.

The subject application plans and reports reflect the endorsed EBV Development Plan, showing the creation of the new ‘Main Street’, including a piazza, and the required connections from Rickard Street, Gamble Street and Elm Grove (subject to development of land to the north), including landscaping.

The western end of ‘Main Street’ is set aside for pedestrian, cyclist and emergency vehicle use only in accordance with the EBV Development Plan.

The internal streets will remain in private ownership as detailed in the EBV Development Plan, but will be available for unrestricted public pedestrian, cyclist and vehicle use (roads only), to be formalised under a Section 173 Agreement with Council. The Agreement will require the owner to indemnify Council against any claims associated with the use of the publicly accessible areas and to accept liability and responsibility for their on-going maintenance.

### Is the building envelope acceptable?

The application plans are accepted as reflecting the building envelopes prescribed under the endorsed EBV Development Plan, which include:

- 5 storey maximum building heights to Nicholson Street incorporating 3 storey street wall heights.

- 5 storey maximum building height to John Street incorporating 4 storey street wall heights.

- 6 storey maximum building heights restricted to the central part of the site.

The separation distances between the buildings, including the dimensions of the areas proposed for public access and the piazza, are in accordance with the EBV Development Plan, and are accepted. They include minimum width dimensions of between 17 and 23.5 metres along ‘Main Street’, and an area set aside for the piazza.

### Has the proposal adequately addressed internal amenity?

The application has been well designed in terms of internal amenity with:

- Minimum 9 metre building separations within internal courtyards (Lots 1, 2, 3, 4 and 7), and staggered / offset living rooms to prevent direct viewing between dwellings.

- No bedrooms relying on borrowed light.

- The majority of dwellings having living areas with depths of less than 8.2 metres.

The recommendation includes minor modifications to dwelling layouts and the inclusion of additional windows in several dwellings in Lots 1, 3, 4, 5, 6 and 7 to improve access to natural light and ventilation.

### Has the proposal adequately addressed external amenity?

The EBV Development Plan requires overlooking and overshadowing to be addressed at the interface with a residential zone. The potentially affected properties are confined to 76 and 78 John Street and 7 and 9 Gamble Street (zoned General Residential) which are adjacent to proposed Lot 5. All other adjacent properties are zoned Commercial 1.

#### Privacy / overlooking

The recommendation addresses overlooking from Lot 5 to the residentially zoned properties in John Street and Gamble Street with requirements for:

- Obscure glazing and maximum awning window openings of 125mm to the south-facing windows of Units 1.24 and 1.25 of Lot 5.

- Solid non-transparent finishes to west-facing balcony balustrades above ground floor to Lot 5, (or partially transparent finishes if utilising patterning / images or lasercut technology). This is proposed in place of the clear glazed balustrades proposed which limit privacy.

While not required to be addressed under the EBV Development Plan, minor modifications to the south elevation of Lot 2 at first and second floor level will ensure that adequate privacy is experienced within dwellings approved at 6-8 Gamble Street. The recommendation includes requirements for:

- Minimum 1.7 metre high screening to the southern end of the first floor common courtyard and balcony perimeters to Units 1.19 and 1.32 of Lot 2.

- Minimum 1.7 metre high screening to the southern end of the bedroom balcony to Unit 1.31 of Lot 2.

- Minimum 1.7 metre high screening to the southern end of the living room balcony to Unit 2.32 of Lot 2.

- Obscure glazing to a minimum height of 1.7 metres to the south-facing windows in the common circulation corridors at first and second floor levels to Lot 2 adjacent to Units 1.31 and 1.32, and Units 2.13 and 2.32.

#### Overshadowing

The EBV Development Plan requires existing solar access to be maintained, or for Clause 55 (ResCode) standards to be satisfied at the interface with a residential zone.

The applicant has provided angled shadow diagrams demonstrating that the existing industrial buildings to the north of the residentially zoned properties in John Street and Gamble Street overshadow the adjacent residences more than the proposed development would. As such, the requirements are met.

### Has adequate car parking been provided?

The car parking provision satisfies the minimum rates endorsed under the EBV Development Plan and is therefore accepted.

| Use | Spaces required under the EBV Development Plan |  | Spaces provided |
| --- | --- | --- | --- |
| Dwellings – 1 bedroom | Not less than 0.64 spaces per dwelling (241): 154 | Dwellings (total): 517 | 525 |
| Dwellings – 2 bedroom | Not less than 1 space per dwelling (360): 360 |  |  |
| Dwellings – 3 bedroom | Not less than 1.5 spaces per dwelling (2): 3 |  |  |
| Shops | Not less than 2.5 spaces per 100 sq m shop area: 113 |  | 223 |
| Total | 630 |  | 748 |

The EBV Development Plan endorsed rates are exceeded for dwellings by 8 spaces and for shops by 110 spaces. The exceeded shop rates will be able to more than adequately cater for car parking for the other commercial uses (food and drink premises and home offices) which require 26 car parking spaces under Clause 52.06 of the Moreland Planning Scheme.

### Has adequate bicycle parking been provided?

The proposed development includes provision of 748 bicycle parking spaces, comprising 614 resident spaces (for 603 dwellings), 48 spaces for retail staff, and 90 spaces at ground level in and around the publicly accessible areas on ‘Main Street’. Three showers with change rooms are provided for retail staff use.

The provision exceeds the minimum rates endorsed under the EBV Development Plan of: 1 space per dwelling; and more than the planning scheme requirements for commercial uses. The commercial requirement under Clause 52.34 of the Moreland Planning Scheme is for 26 bicycle parking spaces (11 staff; 15 visitor / shopper), and 2 showers with change rooms.

### Will the proposal create an excessive traffic impact on the road network?

Council’s Strategic Transport and Urban Safety Branch have reviewed the proposal and confirmed that the traffic generated onto John Street (approximately 800 vehicle movements per day) is commensurate with the anticipated maximum 1200 vehicle movements per day in association with the development of the entire precinct covered by the Development Plan Overlay, as detailed under the EBV Development Plan.

Traffic calming road works in John Street are required to be delivered by the developer at no cost to Council to address additional traffic impacts and minimise amenity impacts on existing residents.

No vehicles will be able to access parking in association with the commercial uses from John Street, in accordance with the endorsed EBV Development Plan.

The use of Rickard Street by delivery trucks in association with the supermarket and shops in Lots 1 and 2 is in accordance with the endorsed EBV Development Plan.

### Has adequate provision for loading been provided?

Dedicated loading is proposed in association with the supermarket. Shared loading bay facilities are provided at ground level in Lots 1 and 4 for use by specialty shops. All loading bays exceed the minimum requirements (area and clearance) under Clause 52.07 of the Moreland Planning Scheme.

The recommendation includes a requirement for a Supermarket Loading and Unloading Management Plan to be submitted to Council’s satisfaction prior to the commencement of the supermarket use to address restricted hours, maximum vehicle length requirements (to negotiate Rickard Street), and measures to prevent conflict with pedestrians, cyclists and other vehicles.

The recommendation also includes the requirement for an acoustic report to be submitted to Council’s satisfaction prescribing specific attenuation measures/treatments to ensure dwelling occupants of the proposed development are provided with an acceptable level of internal amenity in respect to noise emanating from the supermarket (including, but not limited to noise from the loading area).

### Does the proposal incorporate adequate Environmental Sustainable Design features?

Council’s ESD Engineer is satisfied that the development can meet a best practice standard equivalent to 4 Star Green Star, subject to conditions requiring clarification and clear commitments to delivery of initiatives.

Notable initiatives include:

- Thermal chimneys to cross-ventilate all dwellings.

- Double glazing throughout.

- Green walls (Lot 2), rain gardens / bioswales (John Street frontage), and roof / courtyard gardens.

- Rainwater harvesting for all toilet flushing.

- Average First Rate 7.5 Star house energy rating.

- 25 percent energy improvement over minimum building code standards for commercial areas.

- Smart metering to each apartment and retail tenancy.

- Photovoltaic systems on the roof of each building.

### Adaptable dwellings

The application plans include optional adaptable floor layouts for 60 of the 603 dwellings (equivalent to ten percent).

The recommendation includes a requirement for the adaptable dwellings to be provided within the development, (not optional).

Land contamination

The site is affected by an Environmental Audit Overlay (EAO) introduced as part of Amendment C92 to the Moreland Planning Scheme. A condition is therefore recommended requiring an Environmental Audit to be undertaken before the development commences. This will ensure that the site is remediated to an appropriate standard to ensure the land is safe for future residents.

Advice from a consultant environmental scientist has been submitted with the application concluding that, based on the results of a Phase 2 Environmental Site Assessment, the nature and extent of contamination from the site is unlikely to preclude the site from being developed for residential use.

## 5.	Response to Objector Concerns

Nil.

## 6.	Officer Declaration of Conflict of Interest

Council Officers involved in the preparation of this report have no Conflict of Interest in this matter.

## 7.	Financial and Resources Implications

Nil.

## 8.	Conclusion

It is considered that the development, which is generally in accordance with the endorsed East Brunswick Village Development Plan, can be supported subject to conditions to address staging, adverse amenity impacts, developer contributions, best practice ESD, and provision of adaptable housing.

Approval of the application will facilitate the creation of a new centre within the precinct with inherent community benefits.

Recommended conditions will ensure delivery of developer contributions including a signalised intersection at Nicholson Street, traffic calming road works in John Street, a community infrastructure cash contribution, construction and ongoing maintenance of internal streets and a central piazza to be made available for public use.

It is recommended that a Permit issue subject to conditions.

Do not delete this line

## Attachment/s

| 1 | Location Plan - MPS/2013/979 | D14/368307 |  |
| --- | --- | --- | --- |
| 2 | Urban Context - MPS/2013/979 | D15/639 |  |
| 3 | Development Plans - MPS/2013/979 | D15/640 |  |
| 4 | Elevations and Sections - MPS/2013/979 | D15/642 |  |
| 5 | Concept Landscape Plan - MPS/2013/979 | D15/638 |  |
| 6 | Staging Plan - MPS/2013/979 | D15/637 |  |
| 7 | EBV Development Plan Endorsed Building Envelopes | D15/636 |  |