# Green Infrastructure Design Checklist

## For use with Guidelines for Green Infrastructure

### Published in partnership by City of Melbourne and Merri‑bek City Council

May 2026

This Checklist details relevant planning and design considerations when incorporating GI onto a new development, following City of Melbourne and Merri-bek City Council’s Guidelines for Green

Infrastructure from early planning to post-construction and maintenance. For further direction or detail regarding a particular element of a GI asset, see the associated page number in the leftmost column. The rightmost column highlights when qualified personnel may be consulted for advice as needed.

This Checklist is separate from any planning or building permit application requirements. Please determine planning requirements for your development by checking the Planning Scheme or liaising with the relevant Responsible Authority.

Page

Item

Relevant expertise

Site and context

Site area

| 11 | Establish site area for the development | Land Surveyor |
| --- | --- | --- |
| 11 | Determine extent of deep soil (>500mm) on-site | Geotechnical Engineer |
| 11 | Determine soil type present and suitability for in-ground planting | Landscape Architect, Horticulturalist |
| 11 | Test for soil contaminates and pH and amend as required | Soil scientist, Geotechnical Engineer |
| 11 | Identify pre-existing vegetation that can be retained | Horticulturalist |
| 11 | Identify and retain significant trees on-site | Horticulturalist/Arborist |
| 11 | Determine canopy spread of retained trees & expected canopy spread post-development and maturation of new canopy trees | Horticulturalist |

Local context

| 11 | Locate nearby open space or parklands to inform connections and/ or planting design | Horticulturalist Council Open Space Team |
| --- | --- | --- |
| 12 | Determine potential vegetation overhang to neighbouring properties, including any hazardous vegetation to be removed. | Horticulturalist |
| 12 | Liaise with neighbouring property owners and discuss plans to implement GI |  |

Site constraints

| 12 | Determine level of sunlight available to inform planting design | Landscape Architect |
| --- | --- | --- |
| 12 | Determine the presence of high winds and associated risks | Landscape Architect |
| 12 | Ascertain whether GI assets will be located on private or common property, and appropriate responsibility for maintenance | Building Surveyor Land Surveyor |
| 12 | Determine responsibility for ongoing vegetation maintenance—engage with neighbouring property owners as required | Horticulturalist |

## Buildings and structures

Page

Item

Relevant expertise

— host building

Adjacent structures and equipment

| 12 | (Green roofs) Determine any additional structures and equipment to be co-located with GI asset | Landscape Architect |
| --- | --- | --- |

Weight load

| 13 | Determine weight load of building and applicable typologies | Structural engineer |
| --- | --- | --- |
| 13 | (Green roofs and plants in containers) Determine point loads for roof/balcony surfaces | Structural engineer |
| 13 | (Green roofs and plants in containers) Determine appropriate spaces for locating heavy elements above structural columns | Structural engineer |
| 13 | (Green roofs) Determine applicability of roof weight loads, including point loads for extensive or intensive green roof | Structural engineer |

Fire risk

| 13 | Determine fire risks present on building | Building Surveyor Fire Safety Engineer |
| --- | --- | --- |
| 13 | Consider contribution to fire risk from added greenery | Building Surveyor Fire Safety Engineer |
| 13 | (Green façades and green walls) Locate non-operable and operable windows and ascertain risk of fire spread from vegetation | Building Surveyor Fire Safety Engineer |

## Buildings and structures

Page

Item

Relevant expertise

— Green Infrastructure design elements

Green roofs

| 13 | Determine appropriate system for development—extensive/ intensive | Structural Engineer |
| --- | --- | --- |
| 13 | Determine any arbours, trellis, or other structures to incorporate and structural weight load implications | Structural Engineer |

Green walls

| 13 | Determine connections to drainage points for external green walls | Structural Engineer |
| --- | --- | --- |
| 13 | Determine anchor points for attaching vertical infrastructure and associated weight loads | Architect Structural Engineer |

Green façades

| 13 | Specify products to be used for growing plants—trellis/cable/mesh | Landscape Architect/Horticulturalist |
| --- | --- | --- |
| 13 | Determine anchor points for attaching vertical infrastructure and associated weight loads | Architect Structural Engineer |

Planters on structure

| 13 | Determine size of planters and appropriate plant species/sizes | Landscape Architect/Horticulturalist |
| --- | --- | --- |
| 13 | Locate planters to not impede accessibility to exits | Landscape Architect |
| 13 | Locate structural columns for placement of heavy planters | Structural Engineer/ Landscape Architect |
| 13 | Anchor fixed furnishings onto structure | Landscape Architect |

Page

Item

Relevant expertise

Waterproofing and drainage

Waterproof membranes

| 14 | Determine waterproofing membrane to use and testing method/ system | Waterproofing Expert |
| --- | --- | --- |
| 14 | Specify waterproofing product and design—ensure UV stability and root resistance barrier | Waterproofing Expert |
| 14 | Install waterproofing to AUS/NZ Standard | Waterproofing Expert Master Plumber |

Testing waterproofing

| 14 | Test waterproofing for correct installation and eliminate leaks | Waterproofing Expert |
| --- | --- | --- |

Drainage systems and cells

| 15 | Determine legal point of discharge (LPOD) for drainage system | Master Plumber Council Drainage Team |
| --- | --- | --- |
| 15 | Specify drainage system including any surface drains and pits for additional drainage/access | Master Plumber |

Page

Item

Relevant expertise

Irrigation systems

Irrigation design

| 16 | Determine irrigation demand | Horticulturalist |
| --- | --- | --- |
| 16 | Determine appropriate irrigation methods and systems | Irrigation Specialist |
| 16 | Ensure backflow prevention devices are properly installed | Master Plumber |

Alternative water sources

| 16 | Connect alternative water sources where possible | Master Plumber |
| --- | --- | --- |
| 16 | Ensure alternative water sources are appropriately filtered for use | Master Plumber |

Irrigation frequencies

| 16 | Test irrigation system and calibrate for system | Horticulturalist/ Irrigation Specialist/ Maintenance Coordinator |
| --- | --- | --- |
| 16 | Connect irrigation system to BMS where appropriate | Irrigation Specialist /Maintenance Coordinator |
| 16 | Monitor and calibrate irrigation regime through establishment | Maintenance Coordinator |
| 16 | (Green walls and façades) Determine fertigation system suitability—where alternative water sources are not in use | Irrigation Specialist |

## Planting

| Page | Item | Relevant expertise |
| --- | --- | --- |
| Growing media |  |  |
| 17 | Specify growing media for plantings and timelines for refreshing growing media/replacing planters | Horticulturalist |
| Elevation and aspect |  |  |
| 17 | Determine appropriate plantings for available sunlight and high winds | Horticulturalist |
| Vertical screening |  |  |
| 17 | Determine appropriate evergreen/deciduous species for shade | Horticulturalist |
| Green roof vegetation |  |  |
| 17 | Determine appropriate plant species for system and soil depth | Horticulturalist |
| 17 | Determine long-term design approach and ongoing maintenance | Horticulturalist |
| 17 | Determine appropriate species for visitors if applicable—sensory experiences and allergy considerations | Horticulturalist |

For more information visit: merri‑bek.vic.gov.au/better‑quality‑developments melbourne.vic.gov.au/green‑infrastructure