Moreland Tree Planting Manual for Residential Zones

(22 December 2014)

1.0 Introduction

This Manual has been prepared to provide guidance to planning permit applicants about the recommended size and species of trees to be included in landscape plans to meet the landscape requirements of the schedules to the Residential Zones. The information contained in this Manual is based on the Moreland Street Landscape Strategy (adopted 8 August 2012).

Moreland’s schedules to the residential zones require the planting of suitable trees in certain locations, including:

- Front setbacks

- Side and rear setbacks

- Secluded private open space

Depending on the specific schedule, tree planting will be required in at least one of the above locations. The requirements of the relevant schedule should be referred to for confirmation of the extent of tree planting to be confirmed on landscape plans. Landscape plans are required to be submitted with permit applications.

2.0 Matching Tree Size to Available Space

The space available for tree planting will differ depending on the size of building setbacks from property boundaries and the size of secluded private open space.

The matrix in figure 1 below should be used to guide whether a large, medium or small tree should be selected. Once size of tree is determined, the tree species selection can be made (refer section 3.0 and the recommended species lists in Appendix 1).

The Tree Planting Space Matrix defines the space available to grow a tree and fits the most appropriate sized tree into the space based on the fundamental design parameter of using the largest acceptable tree in each planting situation.

The Tree Planting Space Matrix combines the width and depth of the space available for the tree to provide sufficient space for the trunk and main structural roots and for tree canopy growth. It assumes the same amount of space is available at ground level and canopy level.

A minimum planting bay of 600mm is required.

Figure 1. Tree planting space matrix

|  |  | Width of available space |  |  |
| --- | --- | --- | --- | --- |
|  |  | > 4 metres | 3-4 metres | 1.8-3 metres |
| Depth of available space (eg. front setback distance) | > 4 metres | L | L or M | VS |
|  | 3-4 metres | S or M | S or M | VS |
|  | 1.8-3 metres | VS | VS | VS |

Note. Where there is less than 1.8m dimension, there is insufficient room for tree planting.

2.1 Tree Size

Four basic tree size categories were developed based on the actual dimensions of available or commonly used trees and the space typically available.  The four size categories are:

1.	Very small: 6 or less metres tall by 6 or less metres wide

2.	Small: 6 to 8 metres tall by 5 to 10 metres wide

3.	Medium: 8 to 15 metres tall by 8 to 18 metres wide

4.	Large: 15 or more metres tall by 15 or more metres wide

Tree sizes at maturity have been estimated.  Obviously tree growth varies from one individual to another and between populations and different provenances, so wherever possible sources of trees have been identified to reduce the likelihood of trees growing much larger than anticipated.

3.0 Recommended Tree Species

The tree size matrix in section 2.0 above should be used to determine what sized tree is suitable for the available space. Once the right size is determined, the species can be selected. The recommended species lists in Appendix 1 are grouped into different sized trees.

The recommended species include a combination of indigenous, Australian native and exotic tree species that have shown to perform well in Moreland. It provides a basis for selection but is not intended to be exhaustive.

Selection of appropriate species should also consider site orientation and whether deciduous or evergreen species are more suitable to allow for summer sun and winter shading to north and west facing windows and open space.

Trees considered to be environmental weeds must not be planted. Species with weed potential or a tendency to sucker should not be planted within creek buffer zones or other environmentally sensitive areas. This applies to the following species in Appendix 1:

- Celtis australis and C. occidentalis

- Robinia pseudoacacia

- Gleditsia triacanthos

- Olea europaea

- Casuarina cunninghamiana

- Ulmus procera

- Platanus xacerifolia

Trees must be able to survive with natural rainfall. Reliance on irrigation systems is not supported.

4.0 Tree Planting and Establishment Methods

4.1 Tree Planting Details

Trees at planting should be at least:

- Height – 2 metres

- Caliper width – 30mm

- Rootball - must provide enough ballast to support the tree

- A minimum planting bay of 600mm is required

- Tree should be staked on both sides for support

Refer to diagram on following page.

Other notes to consider include:

- Opportunities for passive watering (e.g. can it be connected to nearby house drain downpipes?)

- Planting configuration and context (how do proposed trees integrate with overall landscape plan onsite and in nearby public realm?)

- Consideration of underground and overhead services.

4.2 Tree Maintenance

Landscape Plans should include an establishment maintenance plan. An example of a two year establishment maintenance plan is shown in the table below.

| Activity | Year 1 – Number of visits | Year 2 – Number of visits |
| --- | --- | --- |
| Watering | 30 | 20 |
| Weed control | 2 | 1 |
| Mulch top up | 2 | 1 |
| Formative pruning | - | 1 |
| Stake removal | - | 1 |
| Water well removal | - | 1 |
| Tree tie removal | - | 1 |

The major limitations to root spread will be soil compaction from development works.

The planting hole can be dug with a number of different pieces of equipment.  Recently the use of chain trenchers has become common. These devices will dig a hole of this type readily, in most soils, and will leave the excavated soil in a good tilth condition for backfilling after planting.  In the sandy and loam soils in Moreland holes can be dug at most times of the year.  In the basalt clay soils holes are best pre-dug during summer while the soil is dry.

Unamended site soil is recommended as backfill in all planting situations.  Organic and other amendments are generally of no value and may make the planting environment worse. If organic amelioration of site soil was desired, shallow incorporation of composted organic matter (surface 100 mm only) is recommended. Surface application of organic mulches is a preferred option.

If site soil cannot be used for backfilling the planting hole, a low organic matter sandy soil is recommended.  The drainage rate of this soil should be no less than 5 mm /h.

Drainage will not be required in most planting holes unless the tree is not suited to the drainage characteristics of the site or is of a very large size requiring excavation into underlying clay subsoil.  Drainage will be required where trees are being planted into rain garden planters for water quality treatment of runoff water.

Improving soil conditions for trees growing in paved areas

One simple way of improving tree soil conditions is the use of raised planters that provide an improved soil volume in the root flare area and zone of rapid taper.  This approach also reduces soil compaction around the base of the tree by reducing pedestrian access. If such an approach is used it is essential that the soil in the raised planter is connected to a larger soil volume below the paving.

Structural soil cells (e.g. like ‘stratacell’) should also be considered. They provide volume beneath paving and avoid clutter caused by raised pavers.

Appendix 1. Recommended Species List