SCHEDULE 2 TO CLAUSE 32.07 RESIDENTIAL GROWTH ZONE

--/--/20--

Proposed C153

Shown on the planning scheme map as RGZ2.

ACTIVITY CENTRES – THREE (3) STOREY AREAS

1.0	Requirements of Clause 54 and Clause 55

--/--/20--

Proposed C153

|  | Standard | Requirement |
| --- | --- | --- |
| Minimum street setback | A3 and B6 | Where the setback required by A3 and B6 is greater than 3 metres, the setback from the front street should be 3 metres. Verandahs, pergolas and balconies should not encroach into the setback. |
| Site coverage | A5 and B8 | None specified |
| Permeability | A6 and B9 | None specified |
| Landscaping | B13 | A minimum of one tree should be planted in the front setback, in accordance with the Moreland Tree Planting Manual for Residential Zones, 2014. |
| Side and rear setbacks | A10 and B17 | None specified |
| Walls on boundaries | A11 and B18 | None specified |
| Private open space | A17 | None specified |
|  | B28 | Where a balcony is the only secluded private open space provided, it must consist of a minimum 8 square metres with a minimum width of 2 metres. The minimum area must be unencumbered by air conditioners and other building services. This replaces the balcony dimensions included in B28. |
| Front fence height | A20 and B32 | None specified |

2.0	Maximum building height requirement for a dwelling or residential building

--/--/20--

Proposed C153

A building used as a dwelling or a residential building must not exceed a height of 10.5 metres (3 storeys) unless the slope of the natural ground level at any cross section wider than 8 metres of the site of the building is 2.5 degrees or more, in which case the height of the building must not exceed 11.5 metres.

This does not apply if the average height of buildings on abutting allotments facing the same street is greater than 10.5 metres, in which case the average height of abutting buildings must not be exceeded.

This does not apply if an Overlay or Council endorsed Structure Plan includes heights greater than 13.5 metres, in which case the height in the Overlay or Structure applies.

3.0	Application requirements

--/--/20--

Proposed C153

The following application requirements apply to an application for a permit under clause 32.07, in addition to those specified in clause 32.07 and elsewhere in the scheme:

- An existing vegetation plan that shows any proposed tree removal.

- A landscaping works and planting plan that includes information on tree selection and planting, in accordance with the landscaping requirements of clause 1.0 of this schedule and the Moreland Tree Planting Manual for Residential Zones, 2014.

4.0	Decision guidelines

--/--/20--

Proposed C153

The following decision guidelines apply to an application for a permit under clause 32.07, in addition to those specified in clause 32.07 and elsewhere in the scheme:

- Whether the development provides an appropriate transition to properties in an adjoining zone.

- The requirements of clause 22.01 Neighbourhood Character Local Policy.

- The requirements of clause 22.08 Development of Four or More Storeys.

- The Moreland Tree Planting Manual for Residential Zones, 2014.